BAS051 - License and Disclaimer Statement

Copyright (c) 1989 W. Washington

All rights reserved.

BAS051 is distributed as a user supported software product.  You may copy and

distribute BAS051 and its accompanying files freely, as long as the files are

distributed together unaltered.  There is NO registeration fee to use BAS051 in

noncommerical applications.  Permission for commerical applications can easily

be obtained from the author.  The author reserves the exclusive right to

distribute BAS051 for profit.  BAS051 is written in C and a diskette containing

the source for BAS051 is availiable from the author by sending $10 to the

address below:

Winefred Washington

P. O. Box 3206

Huntsville, AL 35810

BAS051 is distributed "as is" without warranty as to its performance, or

fitness for any purpose.  The entire risk is assumed by the user.

Table of Contents

I.         Introduction

II.        BAS051 syntax

III.       Compiling a BASIC program

IV.        BAS051 Supplied Routines and Libraries

V.         The Author's Tips and Comments

I.         Introduction

BAS051 - A BASIC Compiler for the 8051 Family

BAS051 is a BASIC compiler for the 8051/8031 microprocessor family which runs

on an IBM PC or compatiable equipped with MS-DOS 2.0 or higher.  This compiler

recognizes a subset of the BASIC programming language and generates assembly

language sources files.  These files are input to an assembler to generate the

machine language bytes for an EPROM.

In addition to BAS051, the following files for included:

BAS051.exe  -     the BASIC compiler executable

BAS051.doc  -     BAS051 documentation

example.lib -     an example library for BAS051

example.bas -     an example BASIC program

example.asm -     the compiled .asm output

example.lst -     the listing file from the assembler

example.hex -     the Intel Hex format of the example program

II.        BAS051 Syntax

BAS051 recognizes a subset of the BASIC programming language.  Limited error

checking is performed; therefore, it is important to submit syntactically

correct programs for BAS051 to compile.  BAS051 is described below in Backus

Naur Form (BNF).  All BASIC keywords appear in uppercase, and non-terminals

appear in lowercase.  ASM and MEM are additional keywords added to enhance

BAS051 programs.

ASM was added to allow the user to insert assembly language source lines inside

the compiled code.  The ASM command enables the application program to be "fine

tuned" by using assembly language statements.

MEM is used to access external RAM.  When MEM is compiled a MOVX instruction is

generated with the DPTR pointing to the address given by the MEM function.  By

using MEM, the user can access up to 64K of external RAM.

BAS051 is not case-sensitive, so keywords and variable names such as "THEN" and

"then" are the same.  Variable name lengths are limited by the assembler used

to generate the machine code, also all variables are one (1) byte.  In larger

systems, the one variable/one byte feature is a restriction; however, in

applications which are not math intensitive, the feature allows BAS051 to

generate faster code.

BAS051 BASIC Syntax in BNF

program          : statements

;

statements       :

| statement statements

;

statement        : Number instruction ;

instruction      : Variable = expression

| MEM [ expression : expression ] = expression

| FOR Variable = expression TO expression

| Next

| IF ( expression relop expression ) THEN Number

| GOTO Number

| GOSUB Number

| RETURN

| READ Variable rest_of_read

| PRINT expression rest_of_print

| TITLE Textstring semicolon

| REM Textstring

| ASM Textstring

| END

;

expression       : Variable

| MEM [ expression : expression ]

| Number

| ( expression )

| expression + expression

| expression - expression

| expression * expression

| expression / expression

| expression MOD expression

| expression AND expression

| expression OR expression

| expression XOR expression

;

relop            : >

| <

| =

;

rest_of_read     :

| , Variable rest_of_read

;

rest_of_print    :

| , Variable rest_of_print semicolon

;

semicolon        :

| ';'

;

III.       Compiling a BASIC program

The command syntax of BAS051 is given below:

Syntax:          bas051 [option] file

Flags:           /c    The flag /cnnnn sets the origin of the code segment

to nnnn.

/d    The flag /dnnnn sets the origin of the data segment

to nnnn.

/l    The flag /llibname adds the library name given by

libname to the list of libraries added to the .asm

file.  An program may contain up to four libraries.

Examples:        bas051 /lexample.lib example.bas

bas051 /d100 /c0xaaa /lstdlib.h c:\basic\prog.bas

^       ^      ^

|       |      |

|       |      --- library of routines

|       |

|       ------ hexidecimal code origin

-------------- decimal data origin

V.         BAS051 Supplied Routines and LIbraries

Commonly used routines can be placed in libraries which are added to BAS051

compiled programs.  Example.lib is a library included to illustrate the

technique used.  Below is a list of the routines which must be included with

every program compiled under BAS051:

basint: -   initialization code

print: -    output character routine 9600 baud

read: -     input character routine

At the start of every compiled program a long call to BASINT is made.  All

additional initialization is placed in this routine by the user.

When input/output is required in a BAS051 program the compiler generates long

calls to the "print" and "read" routines which send and receive data from the

8051's serial port.  If the target has a different I/O system, the user may

rewrite these routines to handle the custom I/O requirements.

The user should only modify these routines once he/she has a firm understanding

of BAS051s inner workings.

V.         The Author's Tips and Comments

Listed below are helpful hints and suggestions about BAS051.

A.   Variables in BASIC programs have a period appended to them

when they appear in the .ASM file.

Example: "count" becomes "count."

B.   Line numbers in BASIC program have ".L" prefixed to them

when they appear in the .ASM file.

Example: "1000" becomes ".L1000"

C.   Variables and line numbers may be accessed by custom assembly

language routines by using their BAS051 generated names.

To call a BASIC subroutine located at line 1000, simply write

"ASM lcall   .L1000".

D.   The assembly language output from BAS051 is based on the

UNIX System V* assembler syntx.  The product, A51, by

PseudoCode is an excellent assembler, reasonably priced,

and can accept BAS051 .ASM files without modification.

A51 is availiable from MicroKit, their address is:

MicroKit

6910 Patterson SE

Caledonia, MI  49316

(616) 791-9333

E.   BAS051 opens a .tmp file called filename.tmp where filename

equals the name of the BASIC program being compiled.  This

file is deleted at the end of the compilation; therefore,

the user should not use this filename for anything since it

is overridden and deleted.

F.   Since BAS051 opens several files at once, the user's CONFIG.SYS

file should contain the line "FILES = xxx", where xxx is a

value indicating the number of files that can be opened at the

same time.  A value of ten is a good setting although BAS051

does not open that many files.

G.   Helpful suggestions and comments are welcome and can be

sent to the address given below.  If you would like the source

to BAS051, just send $10 and you will receive a 5 1/4"

floppy containing the source used to create BAS051.

Winefred Washington

P. O. Box 3206

Huntsville, AL 35810

H.   If reader interest and support are great enough, future versions

of BAS051 will include:

1.  more variable types and sizes

2.  an intergrated assembler

3.  a linker and librarian.

* UNIX is a trademark of AT&T.